# FPGA Hardware Interface Structure

WiMAX IEEE 802.16-2007 PHY Layer Transmitter

Target Board: DE0-CV 5CEBA4F23C7 (Cyclone V FPGA)

Contents

Buttons (Active-Low)	3

KEY0 — Reset	3

KEY1 — Load/Start	3

Switch	3

SW0 — Mode Select	3

LED Indicators	3

Pass/Fail Status LEDs (LEDR[3:0])	3

Verification Complete LEDs (LEDR[7:4])	4

System Status LEDs (LEDR[9:8])	4

Verification Method	4

Operation Summary	4

Expected Successful Operation	4

Pin Assignment Summary	5

Credits	5

# Buttons (Active-Low)

## KEY0 — Reset

- Function: System reset

- Behavior: Press to reset the entire system. Clears all verification states, counters, and latched status registers. Prepares for a new verification cycle.

- Pin Assignment: PIN_U7

## KEY1 — Load/Start

- Function: Initialize PRBS and start transmission

- Behavior: Press to load the PRBS seed and begin processing data through the WiMAX PHY pipeline. Triggers verification of 5 consecutive packets against golden reference data.

- Pin Assignment: PIN_W9

- Note: Button is debounced (~10ms) to prevent multiple triggers.

# Switch

## SW0 — Mode Select

- Pin Assignment: PIN_U13

- LOW (0): Single-shot mode — Processes and verifies 5 packets, then automatically stops. Enable signal (LEDR[8]) turns off after verification completes.

- HIGH (1): Continuous mode — Continuously processes packets indefinitely. Useful for long-term stability testing.

# LED Indicators

The DE0-CV board provides 10 LEDs (LEDR[9:0]) for status indication. All LEDs are active-HIGH (ON = logic 1).

## Pass/Fail Status LEDs (LEDR[3:0])

These LEDs indicate whether each processing stage passed verification across ALL 5 packets. They start ON (assuming pass) and latch OFF if ANY packet fails.

| LED | Signal | Description |
| --- | --- | --- |
| LEDR[0] | PRBS Pass | ON: All 5 packets from randomizer match golden data OFF: At least one packet failed verification |
| LEDR[1] | FEC Pass | ON: All 5 packets from FEC encoder match golden data OFF: At least one packet failed verification |
| LEDR[2] | Interleaver Pass | ON: All 5 packets from interleaver match golden data OFF: At least one packet failed verification |
| LEDR[3] | Modulator Pass | ON: All 5 packets from QPSK modulator match golden data OFF: At least one packet failed verification |

## Verification Complete LEDs (LEDR[7:4])

These LEDs indicate when verification has completed for each processing stage (all 5 packets processed).

| LED | Signal | Description |
| --- | --- | --- |
| LEDR[4] | PRBS Verified | ON: All 5 PRBS/randomizer packets have been verified |
| LEDR[5] | FEC Verified | ON: All 5 FEC encoder packets have been verified |
| LEDR[6] | Interleaver Verified | ON: All 5 interleaver packets have been verified |
| LEDR[7] | Modulator Verified | ON: All 5 modulator packets have been verified |

## System Status LEDs (LEDR[9:8])

| LED | Signal | Description |
| --- | --- | --- |
| LEDR[8] | Enable Active | ON: System is actively processing data OFF: System idle or verification complete (single-shot mode) |
| LEDR[9] | PLL Locked | ON: PLL is locked, 50 MHz and 100 MHz clocks are stable OFF: PLL not yet locked, system not ready for operation |

# Verification Method

The system uses golden data comparison to verify correct operation of each processing stage:

- Golden reference data is defined in wimax_pkg.sv

- Each stage output is compared bit-by-bit against expected values

- Verification runs for 5 consecutive packets (configurable via NUM_PACKETS parameter)

- Pass LEDs remain ON only if ALL 5 packets pass for that stage

- QPSK modulator verification compares I/Q sign bits against expected constellation mapping

# Operation Summary

- Press KEY0 to reset the system

  - LEDs 0-3 remain ON after pressing KEY0 (reset) because the design correctly resets the pass-latched signals to their initial PASS state (1'b1), as intended for "assume pass until proven otherwise" logic.

- Wait for LEDR[9] to turn ON (PLL locked)

- Set SW0 to desired mode (DOWN for single-shot, UP for continuous)

- Press KEY1 to load PRBS seed and start transmission

- Observe LEDR[8] turn ON (enable active)

- After verification completes (all 5 packets processed):

- LEDR[7:4] turn ON (verification complete for each stage)

- LEDR[3:0] indicate pass/fail status:

- All 4 LEDs ON = SUCCESS (all stages passed)

- Any LED OFF = FAILURE (that stage had at least one packet fail)

- LEDR[8] turns OFF in single-shot mode (enable deasserted)

## Expected Successful Operation

When the design is functioning correctly, after pressing KEY1:

- LEDR[9] = ON (PLL locked)

- LEDR[8] = ON during processing, OFF after completion (single-shot)

- LEDR[7:4] = All ON (all stages verified)

- LEDR[3:0] = All ON (all stages passed)

# Pin Assignment Summary

| Signal | Pin | I/O Standard | Function |
| --- | --- | --- | --- |
| CLOCK_50 | PIN_M9 | 3.3-V LVTTL | 50 MHz Clock Input |
| KEY0 | PIN_U7 | 3.3-V LVTTL | Reset (Active-Low) |
| KEY1 | PIN_W9 | 3.3-V LVTTL | Load/Start (Active-Low) |
| SW0 | PIN_U13 | 3.3-V LVTTL | Mode Select |
| LEDR[0] | PIN_AA2 | 3.3-V LVTTL | PRBS Pass |
| LEDR[1] | PIN_AA1 | 3.3-V LVTTL | FEC Pass |
| LEDR[2] | PIN_W2 | 3.3-V LVTTL | Interleaver Pass |
| LEDR[3] | PIN_Y3 | 3.3-V LVTTL | Modulator Pass |
| LEDR[4] | PIN_N2 | 3.3-V LVTTL | PRBS Verified |
| LEDR[5] | PIN_N1 | 3.3-V LVTTL | FEC Verified |
| LEDR[6] | PIN_U2 | 3.3-V LVTTL | Interleaver Verified |
| LEDR[7] | PIN_U1 | 3.3-V LVTTL | Modulator Verified |
| LEDR[8] | PIN_L2 | 3.3-V LVTTL | Enable Active |
| LEDR[9] | PIN_L1 | 3.3-V LVTTL | PLL Locked |

# Credits

Made by Team 1:

- Abanoub Emad

- John Fahmy

- Omar ElShazli

Supervised by: Dr. Ahmed Abou Auf

Course: ECNG410401 ASIC Design Using CAD

Date: December 2025